WiFiGate — Technical Handbook

WiFiGate — Technical Handbook

# WiFiGate Handbook — End-to-End Professional Guide

A complete learning document for the WiFiGate system: how everything works, why it is built this way, which libraries are used, all the flows, and the important logs for each stage. Written by reading the actual source code, not from memory.  If you want a general overview, read README.md first. This document is a deeper layer: a learning guide + reference.  🇮🇱 A Hebrew version of this handbook is available at WIFIGATE_HANDBOOK_HE.md.

────────────────────────────────────────────────────────────

## Table of Contents

Big Picture: what WiFiGate is and why

The tech stack — every library and why it was chosen

Code map — where everything lives

Data model — Gate, Role, Group, Dual Control

The storage layer — why local-first and how it works

State management — the custom store

The core end-to-end flows

The secure BLE protocol SECURE_V3 — in depth

Invitation-link encryption — in depth

Deep Linking — how a link becomes a screen

Auto Open — proximity-based opening

FOTA / OTA — firmware update

Nearby Wi-Fi Setup — the local setup page

Important logs — full catalog by tag

Permissions and platform

Secrets and build inputs

Local development, checks, and release

Security boundaries — what is protected and what is not

Cheat Sheet — quick references

────────────────────────────────────────────────────────────

## 1. Big Picture

WiFiGate is a React Native / Expo app for controlling physical gates driven by WiFiGate firmware (based on the ESP32-C6). The app is essentially two things at once:

A gate wallet on the phone — a list of gates with different access levels

and lifetimes.

A secure control client for the device — it opens a gate over Bluetooth

Low Energy using an encrypted protocol.

### The app's three jobs

The app stores gate metadata locally (local-first) and uses that list to drive three jobs:

Opening a gate over BLE (the core).

Sharing / receiving access via encrypted invitation links.

Entering local setup over Wi-Fi (the http://192.168.4.1/ page).

### Guiding principle: separation of security layers

This is a critical point for understanding the whole architecture — there are two separate security problems with two separate solutions:

| Layer | What it protects | Mechanism |
| --- | --- | --- |
| BLE (SECURE_V3) | phone↔gate communication | ECDH + HKDF + AES-256-GCM + HMAC |
| Invitation links | the payload inside the URL | AES-256-GCM with a key derived from HMAC_URL_SECRET |

They do not replace each other. An encrypted link does not protect BLE, and SECURE_V3 does not protect the URL. See Section 18.

┌───────────────────────────────────────────────────────────────┐
│                        WiFiGate app                            │
│                                                                │
│   UI (app/ - Expo Router)                                      │
│        │                                                       │
│   openGateFlow  ─┬─ gatesStore (state)  ─── storage (AsyncStorage)
│        │         │                                             │
│   ble/openGate ──┴─ ble/secureProtocol (crypto SECURE_V3)      │
│        │                                                       │
└────────┼───────────────────────────────────────────────────────┘
         │ BLE (Nordic UART Service)          │ Wi-Fi AP 192.168.4.1
         ▼                                     ▼
   ┌──────────────┐                     ┌──────────────┐
   │ ESP32-C6 gate│◄── OTA (Firebase) ──│  setup page  │
   └──────────────┘                     └──────────────┘

────────────────────────────────────────────────────────────

## 2. The Tech Stack

Source: package.json. Below are the significant libraries and why they are there.

### Core — Runtime and Navigation

| Library | Role | Why |
| --- | --- | --- |
| expo ~54 / react-native 0.81 / react 19 | the base | Expo provides a dev-client, builds (EAS), and config plugins without touching native code by hand |
| expo-router ~6 | file-based routing | every file in app/ is a route; deep-linking is built in |
| expo-dev-client | custom dev build | required — the app depends on native modules (BLE, Wi-Fi) that don't exist in Expo Go |
| react-native-reanimated / gesture-handler / worklets | animations and gestures | smooth UI, drag-to-reorder |
| zustand | state (available) | Note: the main gate store (gatesStore.ts) is hand-built and does not use zustand. See Section 6 |

### Cryptography — the heart of security

| Library | Role |
| --- | --- |
| @noble/curves | P-256 curve (ECDH for the handshake) |
| @noble/hashes | SHA-256, HKDF, HMAC |
| @noble/ciphers | AES-256-GCM |
| react-native-get-random-values | polyfill for crypto.getRandomValues (must load first — see line 1 of app/_layout.tsx) |
| buffer / base-64 | base64 encoding for BLE and links |

@noble/* were chosen because they are pure-JS crypto libraries, no native dependency, audited, and suitable for React Native (which lacks a full crypto).

### BLE and Wi-Fi — the physical transport

| Library | Role |
| --- | --- |
| react-native-ble-plx | the BLE core: scan, connect, GATT, notify/write |
| react-native-bluetooth-state-manager | Bluetooth state (on/off) and enable request |
| react-native-wifi-reborn | scan/connect to the gate's AP (the Nearby setup flow) |
| react-native-webview | displaying the local 192.168.4.1 setup page |

### Identity and Auth

| Library | Role |
| --- | --- |
| @react-native-firebase/auth + firebase | SMS OTP phone authentication |
| @react-native-google-signin/google-signin | Google identity (fills email for BLE) |
| expo-apple-authentication | Sign in with Apple |
| expo-auth-session | OAuth flows |
| libphonenumber-js + country-list | normalizing/validating phone numbers and country codes |

### I/O and UI

expo-camera (QR scanning), react-native-qrcode-svg (QR generation for sharing), expo-contacts (picking an invitee from contacts), expo-file-system + expo-sharing (CSV / file export), react-native-draggable-flatlist (reordering gates), lottie-react-native (animations), @react-native-async-storage/async-storage (local storage).

────────────────────────────────────────────────────────────

## 3. Code Map

app/                         # Expo Router — each screen is a route
  _layout.tsx                # root: deep-link routing, auth gating, splash, auto-open boot
  index.tsx                  # home screen: gate list, groups, search, reorder
  scan.tsx                   # QR scanning + import
  gate/[id].tsx              # link import + gate details screen
  open-gate/[id].tsx         # quick open (from widget/deep link)
  group/[id].tsx             # group screen
  dual-control/[id].tsx      # dual-control screen
  nearby-wifigate-link.tsx   # entry to Nearby Wi-Fi setup
  wifigate-setup.tsx         # the WebView of the local setup page
  firmware-update.tsx        # OTA screen
  settings/[id].tsx          # gate settings
  auth/login.tsx             # login (provider)
  auth/phone.tsx             # phone OTP

src/
  models.ts                  # all types: Gate, UserRole, GateGroup, DualControl...
  storage.ts                 # AsyncStorage — all keys, load/save, normalization
  gatesStore.ts              # the central state store (manual observable) + merge rules
  gateService.ts             # guest-access validation and service logic
  openGateFlow.ts            # the gate-open orchestration (permissions→BT→BLE)
  gateOpenBlockReason.ts     # mapping error messages to block reasons
  gateSecurity.ts            # parsing/normalizing the security bundle (SECURE_V3)

  ble/
    openGate.ts              # the BLE core (scan/connect/handshake/frames) — the big file
    secureProtocol.ts        # the SECURE_V3 crypto (handshake, keys, frames)
    bluetoothState.ts        # Bluetooth state + ensureBluetoothReady
    permissions.ts           # BLE permissions

  inviteLinkCrypto.ts        # encrypt/decrypt of link payloads (AES-GCM)
  linkService.ts             # parse/build invitation links (single/group/dual)
  dualControlInviteImport.ts # import of a dual-control invitation
  groupInviteImport.ts       # import of a group invitation

  auth/                      # LocalAuthContext, providerIdentity, google/apple
  auto-open/                 # runtime + native bridges + headless task + config
  fota/                      # publicFota (Firebase), driveFota (legacy)
  widgets/                   # gate widget (Android) + headless open
  wifi/                      # useGateWifiConnectionGuard
  crypto/                    # hmacSecrets (reconstructs HMAC_URL_SECRET)
  i18n/                      # languages and translations
  permissions/               # appLaunch, contacts, wifi
  utils/                     # datetime, gateCommand, guestAccess, subscription...
  components/                # all UI: modals, gate-settings, admin-access...
  features/my-gates/         # the home-screen collections (tiles, helpers)

docs/
  ble-security-protocol-v3.md  # the formal SECURE_V3 spec
  WIFIGATE_HANDBOOK_EN.md      # this document
  WIFIGATE_HANDBOOK_HE.md      # Hebrew version

README.md                          # general overview
README_WIFIGATE_LINK_INVITATIONS.md# the full invitation-link protocol
README_OTA_FIREBASE.md             # OTA from Firebase Storage
WIFIGATE_API_README.md             # links issued by the WIFIGATE API

────────────────────────────────────────────────────────────

## 4. Data Model

Source: src/models.ts.

### 4.1 Gate

The central entity. Important fields:

type Gate = {
  id: string;            // local identifier (= systemId currently)
  systemId: string;      // hardware id, used in BLE and links. e.g. "9876543210123456"
  systemName: string;    // the original/imported name
  displayName: string;   // the name the user can edit
  role: UserRole;        // ★ the role decides everything (see 4.2)
  subscription?: "OFFLINE" | "ONLINE" | "WIFI" | "ALL_IN_ONE";
  security?: GateSecurityConfig;   // the SECURE_V3 bundle
  gateCommand?: GateCommandType;   // the command type the firmware reported (PULSE/TOGGLE/PUSH_BUTTON...)
  autoOpen?: GateAutoOpenSettings;
  // sharing/hosting:
  hostPhone/hostName/hostEmail?    // the inviter
  guestPhone?                      // the invitee (in a private guest link)
  accessStart/accessEnd?           // validity window (ms)
  // registration for PRE_USER/PRE_ADMIN:
  residentName/floorNumber/apartmentNumber/parkingNumber?
  openHistory?: GateOpenHistoryEntry[];  // local log of recent opens (newest-first)
};

### 4.2 Role — the role decides everything

type UserRole = "ADMIN" | "USER" | "GUEST" | "PRE_USER" | "PRE_ADMIN";

| Role | Meaning | Source | BLE command after import |
| --- | --- | --- | --- |
| ADMIN | confirmed admin | after firmware confirmation | OPEN / TOGGLE_* / AUTO_OPEN |
| USER | confirmed permanent user | after firmware confirmation | OPEN / TOGGLE_* / AUTO_OPEN |
| GUEST | temporary access with a time window | guest link | OPEN_GUEST / TOGGLE_*_GUEST |
| PRE_USER | invited as user, not yet confirmed | user link (r=PU) | ADD_USER |
| PRE_ADMIN | invited/added as admin, not yet confirmed | QR/manual or admin link (r=PA) | ADD_ADMIN |

**Why PRE_*?** Because the app is local-first — it cannot know that the firmware confirmed you. Until the device confirms, you are in a "pending" state. When ADD_USER/ADD_ADMIN runs successfully against the gate, the firmware confirms, and the onRoleUpgrade callback upgrades PRE_USER→USER / PRE_ADMIN→ADMIN (see maybeUpgradeRole in openGate.ts and the persist in openGateFlow.ts).

### 4.3 Role merge rules (why a gate isn't "destroyed")

Source: addGate in gatesStore.ts. The system protects permanent access from an accidental downgrade:

A permanent gate (ADMIN/USER) is not downgraded to GUEST when importing a

guest link.

A permanent role is not overwritten by a weaker PRE_* role.

A guest gate can be upgraded to a permanent/pending role.

A custom name (displayName) is preserved if a later import brings only a

default name.

### 4.4 GateGroup and DualControl

GateGroup — a purely local UI container. It changes no firmware/BLE

behavior. It supports nesting via parentGroupId. It holds gateIds[] and   dualControlIds[].

DualControl — a pair of gates (firstGateId/secondGateId) with two

action labels (firstLabel/secondLabel, e.g. "Open"/"Close"). Every   dual-control invitation contains exactly 2 gates.

────────────────────────────────────────────────────────────

## 5. The Storage Layer

Source: src/storage.ts. The system is local-first — gate state lives in AsyncStorage, not in the cloud. There is no central "gate ownership" server.

### Storage keys (KEYS)

wifigate:gates                 # the gates array
wifigate:localProfile          # onboarding profile (name/email/phone/provider/terms)
wifigate:phone                 # phone number
wifigate:username              # display name
wifigate:language / :languagePreview
wifigate:gates:order           # display order
wifigate:gates:groups          # groups
wifigate:gates:dualControls    # dual controls
wifigate:gates:toggleRelayStates # last relay state for TOGGLE gates
wifigate:firmware:url          # last firmware URL
wifigate:defaultCountryIso2    # default country code
wifigate:appleIdentityRecords  # map userId→Apple identity (Apple doesn't send email every time)
wifigate:guestTable:*          # guest-table display preferences

### Subtleties worth knowing

**loadGates() normalizes on every load**: it merges openHistory with

lastOpenedAt, normalizes gateCommand, fotaChannel, hostEmail   (lowercase), and guest access.

**getJSON/setJSON swallow errors** and return a fallback — a failed write

won't crash the app (but also won't alert — worth remembering while debugging).

Android limit: DEFAULT_ANDROID_ASYNC_STORAGE_LIMIT_BYTES = 6MB.

debugPrintAllLocalStorage() prints used/remaining + a full dump — an   excellent debug tool.

Cached profile: cachedLocalProfile is loaded once;

loadPhone/loadUsername/loadEmail prefer it.

────────────────────────────────────────────────────────────

## 6. State Management

Source: src/gatesStore.ts.

This is a hand-built observable store — not Redux and not zustand (even though zustand is in the deps):

let _gates: Gate[] = [];
const listeners = new Set<() => void>();
export function subscribe(fn) { listeners.add(fn); return () => listeners.delete(fn); }
function notify() { listeners.forEach(f => f()); }

Every mutation (addGate, updateGateRole, renameGate, ...) works immutably (builds a new array), calls saveGates, notify, and when needed syncGateWidgets() + syncAutoOpen().

### initGates() — what happens at startup (very important)

loadGates() from AsyncStorage.

normalizeGateRoleOnLoad — migration of old guests (isGuest:true →

role:"GUEST").

Comment migration (default).

**Remove old gates without a role** — the model is role-based only now.

notify().

cleanupExpired(true) — removes expired guest gates and shows an Alert.

syncGateWidgets() + syncAutoOpen().

Logs to look for: 🔄 [Store] initGates(), 📦 [Store] N gates loaded, 🧹 [Store] Removed N legacy gate(s), 🧹 [Store] N guest gate(s) removed (expired).

────────────────────────────────────────────────────────────

## 7. The Core Flows

### 7.1 Onboarding / Sign in

Routes: app/auth/login.tsx, app/auth/phone.tsx.

The flow:

The user signs in through a provider (Google/Apple) → a normalized email is

stored in the profile.

Profile name → country code → phone number → accept terms of use.

SMS OTP (Firebase Auth) → confirm code.

saveLocalOnboardingProfile stores everything and syncs phone/name to native

Auto Open.

Why phone + email? In a secure BLE open, the firmware validates the inviter pair (host phone + host email) for guests. So the email is stored and passed in the command.

The gating itself is in _layout.tsx (LayoutNavigator): if not onboarded → redirect to /auth/phone (if there is already a provider identity) or /auth/login.

### 7.2 Adding a gate (QR / Manual)

Manual/QR → addGateFromQrOrManual → role PRE_ADMIN.

A QR that contains a compact security bundle (dpk/psk, without

protocol/keyId) is normalized to SECURE_V3 with keyId="key-1" (see   gateSecurity.ts).

The import/save path rejects a gate without a security bundle — new gates

must have SECURE_V3.

### 7.3 Importing an invitation link (single/group/dual)

Sources: linkService.ts, inviteLinkCrypto.ts, _layout.tsx, [app/gate/[id].tsx](../app/gate/[id].tsx). Full protocol in README_WIFIGATE_LINK_INVITATIONS.md.

The flow:

The OS opens wifigate.io/wifigate-link/ (or -api/) or wifigate://.

_layout.tsx → extractInvitationRoute recognizes the type

(single/group/dual/open-gate).

If not onboarded → the link is queued (pendingInviteUrl) until

onboarding completes.

Route to app/gate/[id].tsx (or group/dual).

parseIncomingLink decrypts ep= (AES-GCM) into compact JSON.

Validation: valid decryption? time window? phone authorization (private

vs public)?

addGateFromGuestInvite / addGateFromUserInvite / addGate (PRE_ADMIN) —

merged into state per the merge rules.

Public vs Private (source: the links protocol):

If field p (target phone) is present → the invitation is private: only

the cached phone matching p can accept.

If p is absent → public: restricted only by the time window (for guests).

Role-code mapping: A→ADMIN, U→USER, G→GUEST, PU→PRE_USER, PA→PRE_ADMIN. In practice: an admin invitation is sent as r="PA", user as r="PU", guest as g=1.

### 7.4 Opening a gate over BLE — the full flow

This is the most important flow. Orchestration in openGateFlow.ts:

runGateOpenFlow(gate)
  ├─ ensureBlePermissionsInteractive()   → "permissions_blocked"
  ├─ ensureBluetoothReady()              → "bluetooth_off"
  ├─ loadPhone()                         → "phone_missing"
  ├─ getGuestAccessValidation(gate)      → "guest_expired"
  ├─ loads email/username/toggleRelayState
  └─ openGateBLE({...}) ──────────────────► ble/openGate.ts
        ├─ (handles onRoleUpgrade → updateGateRole)
        ├─ saves toggleRelayState if gateCommand=TOGGLE
        └─ if OK and not PRE_* → markGateOpened (openHistory)

Status mapping: OK→ok, UNAUTHORIZED→unauthorized, TIMEOUT→timeout, FAIL→fail (and if the message indicates an active event/sensor → event_active/sensor_inactive).

Inside openGateBLE (in ble/openGate.ts):

Scan / Connect — findConnectedGate: scans devices, matches by

systemId (the advertised name), connects. There is a fast profile    (default) and a slow one (slow-android). If there is a preferredDeviceId    (from Auto Open) — it tries a direct connection first.

Discover GATT — requests MTU 517 (Android), discovers the Nordic UART

service:

Service 6e400001-...

RX 6e400002-... (write from app to gate)

TX 6e400003-... (notify from gate to app)

Inbox — createBleTextInbox: assembles messages from chunks by \n,

supports both notify and polling (fallback), and filters local echo.

Handshake SECURE_V3 — client_hello → device_hello → session keys (see

Section 8).

Encrypted command frame — builds the command, adds the authTag,

encrypts and sends (seq=1).

Encrypted response frame — reads, decrypts, returns a

SecureDeviceResponse.

### 7.5 Choosing the command by role and gate type

| State | BLE command |
| --- | --- |
| USER/ADMIN, regular gate | OPEN (or AUTO_OPEN) |
| USER/ADMIN, TOGGLE gate | TOGGLE_OPEN / TOGGLE_CLOSE (by the stored toggleRelayState) |
| USER/ADMIN, PUSH_BUTTON gate | PUSH_BUTTON_HOLD (repeated) + PUSH_BUTTON_RELEASE |
| GUEST, regular gate | OPEN_GUEST |
| GUEST, TOGGLE gate | TOGGLE_OPEN_GUEST / TOGGLE_CLOSE_GUEST |
| PRE_USER | ADD_USER (confirms → USER) |
| PRE_ADMIN | ADD_ADMIN (confirms → ADMIN) |

Guest open additionally passes guestName, inviterName, inviterPhone, inviterEmail — and the firmware validates the inviter pair before opening. If inviterEmail is missing in a guest invitation — the app stops with a message to request a new invitation.

Push button is a special case (source: docs/ble-security-protocol-v3.md): each HOLD frame refreshes the session TTL; the same BLE connection stays open while pressing; seq increments per frame; on finger-up a PUSH_BUTTON_RELEASE is sent. The firmware keeps a 1s watchdog as a fallback.

### 7.6 Sharing access (creating links)

Sources: buildShareData (single), buildGroupShareData, buildDualControlShareData in linkService.ts.

An admin can create: a guest link, a user link, an admin link, and a Nearby Wi-Fi setup link. The payload is built compact (i,n,r,h,he,p,kid, dpk,psk...), encrypted by encryptInviteLinkPayload into ep=v1.<base64url>, and combined into the public URL.

────────────────────────────────────────────────────────────

## 8. The Secure BLE Protocol

Sources: src/ble/secureProtocol.ts + the spec docs/ble-security-protocol-v3.md.

SECURE_V3 is the only supported secure protocol. Both the app and the firmware must use v3.

### 8.1 Provisioning bundle

Every secure gate holds a Gate.security:

{ protocol: "SECURE_V3",
  keyId: string,            // key slot, e.g. "key-1"
  devicePublicKey: string,  // compressed P-256 public key, hex
  pairingSecret: string }   // 32-byte per-gate pairing secret, hex

### 8.2 The handshake — step by step

Constants (secureProtocol.ts): version 3, default freshness 15000ms, labels: wifigate-ble-secure-v3 (transcript), device-hello-proof-v3, command-auth-v3.

app                                          gate (ESP32)
   │  1. client_hello (plaintext JSON)          │
   │  { v:3, type, keyId, sessionId,            │
   │    clientPub, clientNonce }                │
   │ ─────────────────────────────────────────►│
   │                                            │  validates v/keyId/nonce
   │  2. device_hello (encrypted)                │  derives helloKey/helloNonce
   │  { v:3, type, ct, devicePub, nonce,        │
   │    sessionId, sid }                        │
   │◄───────────────────────────────────────── │
   │  ct decrypts to:                           │
   │  { deviceNonce, expiresInMs:30000,         │
   │    keyId, pairingProof }                   │
   │                                            │
   │  3. both sides derive session keys          │
   │  4. encrypted frame (OPEN...) seq=1 ───────►│
   │  5. ◄────── encrypted response frame seq=1  │

### 8.3 Key derivation (key schedule)

ephemeral P-256 keypair (client)
sharedSecret = ECDH(clientSecret, devicePub).X    // 32 bytes (X of the compressed point)

helloKey   = HKDF-SHA256(sharedSecret, salt=clientNonce, info="hello-key|sid|session|keyId", 32)
helloNonce = HKDF-SHA256(sharedSecret, salt=clientNonce, info="hello-nonce|...", 12)

// device_hello decrypts:
plaintext = AES-256-GCM(helloKey, helloNonce, AAD="device_hello|systemId|sessionId").decrypt(ct)

// device identity check (proof-of-possession of pairingSecret):
pairingProof = HMAC-SHA256(pairingSecret, "device-hello-proof-v3|systemId|session|keyId|clientPub|clientNonce|devicePub|deviceNonce")

transcriptHash = SHA256("wifigate-ble-secure-v3|systemId|session|keyId|clientPub|clientNonce|devicePub|deviceNonce")

// session keys (salt = clientNonce || deviceNonce):
clientWriteKey     = HKDF(sharedSecret, salt, "client-write-key|...", 32)
deviceWriteKey     = HKDF(sharedSecret, salt, "device-write-key|...", 32)
clientNoncePrefix  = HKDF(sharedSecret, salt, "client-nonce-prefix|...", 8)
deviceNoncePrefix  = HKDF(sharedSecret, salt, "device-nonce-prefix|...", 8)
authKey            = HKDF(pairingSecret, salt=transcriptHash, "command-auth-key|...", 32)

### 8.4 Encrypted command frame

command = { cmd, systemId, timestamp, freshnessMs:15000, phone, role, name, ... }
commandHash = SHA256(stableJson(command))                       // stable JSON (sorted keys)
authTag = HMAC-SHA256(authKey, "command-auth-v3" || transcriptHash || seq(4B BE) || commandHash)

innerPayload = { authTag, command }
nonce = clientNoncePrefix(8) || seq(4B BE)                       // 12 bytes
ct = AES-256-GCM(clientWriteKey, nonce, AAD="frame|c2d|systemId|session|seq").encrypt(innerPayload)
frame = { v:3, type:"frame", seq, ct }

The response (d2c) is symmetric: deviceWriteKey, deviceNoncePrefix, AAD with d2c. The firmware returns { type:"response", cmd, status:OK|FAIL|UNAUTHORIZED, gateCommand?, relayState?, ... }.

stableJson matters: the command is serialized with alphabetically sorted keys and without undefined (stableStringify), so that the SHA256 is identical on both sides. Reordering fields would break the HMAC. Also, systemId is omitted from the wire command when it can be derived from the session.

### 8.5 Replay and Freshness resistance

Replay resistance comes from multiple layers: a fresh sessionId, fresh clientNonce/deviceNonce, per-frame seq, a transcript-bound authTag, session TTL, and freshness enforcement.

Freshness (firmware): rejects a command if freshnessMs is missing/0, if it exceeds the session TTL, or if the session age is greater than freshnessMs. The firmware logs the requested freshnessMs, the measured age, and the rejection reason.

### 8.6 Device key rotation (recovery)

If the gate returns a devicePub that differs from the stored one but the pairingProof is valid — the app accepts the new key (usedStoredDevicePublicKey=false) and refreshes it after the command succeeds. If decryption fails entirely → a friendly error: *"This gate's secure identity changed. Scan the latest WIFIGATE QR..."* (the device was probably reprovisioned/flash-formatted).

### 8.7 Transport parameters (BLE)

From openGate.ts: requested MTU 517 (attribute capped at 512B), chunk = mtu-3, message timeout 10000ms, framing by \n, write with/without response depending on the characteristic. Special messages (device_hello, frame) are filtered by systemId/sessionId/seq — unexpected messages are skipped with [BLE][SECURE] Ignoring unexpected....

────────────────────────────────────────────────────────────

## 9. Invitation-Link Encryption

Source: src/inviteLinkCrypto.ts.

key = HKDF-SHA256(HMAC_URL_SECRET_bytes,
                  salt="wifigate-invite-link:key-salt",
                  info="wifigate-invite-link:aes-256-gcm", 32)
ciphertext = AES-256-GCM(key, managedNonce, AAD="wifigate-invite-link:v1").encrypt(JSON)
token = "v1." + base64url(nonce||ciphertext)

managedNonce(gcm) — the nonce is randomized and prepended to the ciphertext

automatically.

The output is versioned v1. — allows a future version bump.

The website does not decrypt — it just forwards the ep= to the app.

### Security properties and an important limitation

✅ The payload is opaque in chats/browsers/previews. ✅ Tampering causes decryption failure in the app. ✅ The static website doesn't need to know the plaintext.

⚠️ Limitation: this is encryption with a shared secret embedded in the build — not end-to-end per recipient. Any trusted build that contains the URL secret can decrypt valid links. This is stronger than plain query params, but not equivalent to a server-issued/one-time/recipient-bound token.

────────────────────────────────────────────────────────────

## 10. Deep Linking

Source: extractInvitationRoute in app/_layout.tsx.

Supported schemes: wifigate:// (native) and https://wifigate.io (paths: wifigate-link, wifigate-api).

The function parses a URL and decides on the target by priority order: dual-control → group → open-gate (widget) → gate (including ?ep=, which becomes gateId="invite"). It also supports a nested ?link=.

Subtlety: if the user is not onboarded, the link is queued (pendingInviteUrl) and processed only after onboarding completes (a useEffect on isOnboarded). Duplicates are filtered via lastHandledInviteUrlRef.

Logs to look for: [Linking] event/initial URL received, [Linking] Routing invitation URL, [Linking] Queueing invitation URL until onboarding completes, [Linking] Ignoring unsupported URL.

────────────────────────────────────────────────────────────

## 11. Auto Open

Sources: src/auto-open/runtime.ts, config.ts, nativeAutoOpen.ts, autoOpenHeadlessTask.ts.

The idea: when you approach a gate (RSSI strong enough), the app opens it automatically.

Important architecture: the shared JS runtime is actually a fallback only.

On iOS, if there is a native module → ensureNativeIosAutoOpenRunning and

the JS runtime is disabled.

On Android → ensureNativeAutoOpenServiceRunning (a foreground service)

and the JS runtime is disabled.

The JS runtime (runRuntimeTick) runs only if there is no native — it scans

candidates (scanAutoOpenCandidatesBLE), picks the strongest, and dispatches   to processAutoOpenTrigger.

Why native? Because Auto Open must work in the background — JS in React Native does not run reliably in the background, certainly not for continuous BLE scanning.

startAutoOpenRuntime() is called once from _layout.tsx (a useEffect).

Logs: [AutoOpen][Runtime], [AutoOpen][iOS], [AutoOpen][Android], [BLE][AUTO_OPEN] Candidate scan matched N gate(s).

────────────────────────────────────────────────────────────

## 12. FOTA / OTA

Sources: src/fota/publicFota.ts (Firebase, the new one), driveFota.ts (Google Drive, legacy), app/firmware-update.tsx. Guide: README_OTA_FIREBASE.md.

The firmware is read from Firebase Storage by two dimensions:

QR channel: fotaChannel = dev / prod.

hardware variant: 8MB / 16MB.

⇒ 4 feeds: {dev|prod}-fota/{8mb|16mb}/manifest.json. The app downloads encrypted .wgfota chunks listed in the manifest and validates flashVariantMb before continuing.

The gate-side flow: OTA_OPEN over BLE → the gate returns the AP SSID/password/token → the app connects and streams the firmware. The public config is in app.json (expo.extra.fotaStorage*).

Logs: [FOTA].

────────────────────────────────────────────────────────────

## 13. Nearby Wi-Fi Setup

Sources: src/components/gate-settings/admin-access/NearbyWIFIGATELink.tsx, app/wifigate-setup.tsx.

Lets an admin open the gate's local AP and enter the setup page:

The admin chooses "Nearby WIFIGATE Link".

Request BLE permissions + confirm Bluetooth.

Send WIFI_OPEN over BLE.

If the gate approves → it returns the current SSID/password.

Scan nearby Wi-Fi networks (react-native-wifi-reborn).

Connect to the gate's AP.

Open http://192.168.4.1/ in a WebView.

The WebView is hardened: restricted to the gate's local host, blocks arbitrary external navigation, blocks app-scheme navigation, supports CSV download, and offers reload/recovery.

⚠️ The BLE protocol does not secure this HTTP page — it is a separate local-network workflow.

Logs: [NearbyWIFIGATELink], [WebView], [WiFi].

────────────────────────────────────────────────────────────

## 14. Important Logs

logger.ts defines log/warn that run **only in __DEV__**. But note: many of the critical logs (BLE, Store, Linking) use console.log/console.warn directly, meaning they appear in regular builds too. So the BLE trace is the main field debug tool.

### 14.1 Tag catalog (by frequency)

| Tag | Where | What it tells you |
| --- | --- | --- |
| [BLE] | ble/openGate.ts | scan, connect, MTU, TX/RX chunks, timeouts |
| [BLE][SECURE] | ble/openGate.ts | handshake, client/device hello, encrypted frames |
| [BLE][AUTO_OPEN] | ble/openGate.ts | candidate scan for Auto Open |
| [Store] | gatesStore.ts | load, merge, downgrade prevention, cleanup |
| [GateOpenFlow] | openGateFlow.ts | the orchestration steps of the open |
| [MyGates] | features/my-gates | home screen (collections, tiles) |
| [GateScreen] | app/gate | gate details screen |
| [AutoOpen]* | auto-open/* | runtime, iOS, Android, UI |
| [Linking] | _layout.tsx | receiving deep links and routing |
| [ENCRYPTED_INVITE_PARAM] | linkService | parsing ep= |
| [InviteLinkCrypto] | inviteLinkCrypto.ts | payload decryption failure |
| [FOTA] | fota/* | OTA |
| [NearbyWIFIGATELink] [WebView] [WiFi] | setup flow | Nearby Wi-Fi |
| [GateWidget] [WidgetHeadlessOpen] [WidgetOpenGate] | widgets/* | widget open |
| [storage] | storage.ts | usage, dump, write failures |
| [Contacts] [Camera] | permissions | permissions |

### 14.2 What to look for per flow (debug playbook)

A gate open fails:

[GateOpenFlow] Checking BLE permissions / Bluetooth state
[BLE] Scanning for gate for Nms...
[BLE] Gate not found. Move closer...        ← TIMEOUT (proximity / advertised name)
[BLE] Requested MTU 517, negotiated N
[BLE][SECURE] Sending client_hello. sessionId=...
[BLE][SECURE] Waiting for device_hello...
[BLE][SECURE][DEVICE_HELLO_PARSE] ...        ← bundle mismatch (reprovision?)
[BLE][SECURE] Sending encrypted OPEN frame. seq=1
[GateOpenFlow] BLE result: { status: ... }

Gate identity changed: look for Could not decrypt device_hello with the stored secure bundle / pairing proof mismatch → need to re-scan the QR.

A link import fails: [Linking] Routing invitation URL → [InviteLinkCrypto] Failed to decrypt invitation payload (tampering/bad secret) or an Alert "Invalid invitation / Access expired / Unauthorized".

A guest disappeared: 🧹 [Store] N guest gate(s) removed (expired) — expired at startup.

A downgrade was prevented: [Store] Preventing role downgrade for gate X: ADMIN stays.

────────────────────────────────────────────────────────────

## 15. Permissions and Platform

Configured in app.json and native files. The app needs:

Camera — QR scanning.

Bluetooth — gate communication.

Location (Android) — historically required for BLE/Wi-Fi scanning.

Nearby Wi-Fi devices (Android).

Local network (iOS) — access to 192.168.4.1.

Deep linking is configured for wifigate:// and https://wifigate.io.

Permissions entry point:

src/permissions/appLaunch.ts   (checkAppLaunchPermissions = BLE + Bluetooth ready;   prepareStartupPermissionsForSignIn = contacts + BLE).

Bluetooth state: ble/bluetoothState.ts (ensureBluetoothReady).

BLE permissions: ble/permissions.ts (ensureBlePermissionsInteractive,

hasBlePermissions).

────────────────────────────────────────────────────────────

## 16. Secrets and Build Inputs

Required inputs (some are gitignored):

src/crypto/HMAC_SECRETS.json — a secret. Used to reconstruct

HMAC_URL_SECRET (link encryption) and any legacy URL/HMAC protection. Derived   through src/crypto/hmacSecrets.ts.

GoogleService-Info.plist (iOS) + google-services.json (Android) — Firebase.

EAS: the script scripts/eas-pre-install.js places HMAC_SECRETS.json into the build environment (eas-build-pre-install). Outside the repo, provided via an environment variable/CI.

⚠️ If HMAC_URL_SECRET is not configured — deriveInviteLinkKey throws, and all link encryption/decryption fails.

────────────────────────────────────────────────────────────

## 17. Local Development and Release

From package.json:

npm install
# provide src/crypto/HMAC_SECRETS.json (or HMAC_SECRETS_JSON in EAS)
# make sure Firebase files exist for the environment

npm run start:dev-client   # expo start --dev-client
npm run start:usb          # Android over USB (PowerShell script)
npm run reset:app          # reset Android app state
npm run reset:usb          # reset + relaunch

npm run lint               # eslint
npm run typecheck          # tsc --noEmit
npm run verify             # lint + typecheck ★ run before shipping

### Release checklist (device smoke test)

☐ Sign-in via phone OTP

☐ QR / manual import

☐ Encrypted link import

☐ Guest invite flow

☐ User/admin invite flow

☐ Gate open over BLE

☐ Nearby Wi-Fi setup

☐ CSV export from the setup page (if relevant)

Build and submit with EAS after the smoke test.

────────────────────────────────────────────────────────────

## 18. Security Boundaries

It is important to understand what is not protected:

SECURE_V3 protects the BLE transport only — not the HTTP setup page

(192.168.4.1), not link sharing, and not credential storage on the phone.

Invitation links are encrypted and tamper-resistant, but use a **shared secret

in the build** — not a server-issued/one-time/recipient-bound token.

Gate state is local-first in AsyncStorage — there is no cloud-synced gate

ownership.

Roles are enforced by both app logic and device behavior, but the **ultimate

hardware authorization must be enforced in the firmware** — do not trust the   UI.

────────────────────────────────────────────────────────────

## 19. Cheat Sheet

### Files by task

| Want to understand... | Read |
| --- | --- |
| BLE crypto and message flow | docs/ble-security-protocol-v3.md, secureProtocol.ts |
| BLE core (scan/connect/frames) | ble/openGate.ts |
| open orchestration | openGateFlow.ts |
| role and merge rules | models.ts, gatesStore.ts |
| creating/importing links | linkService.ts, inviteLinkCrypto.ts |
| full links protocol | README_WIFIGATE_LINK_INVITATIONS.md |
| deep linking | app/_layout.tsx |
| Nearby setup | NearbyWIFIGATELink.tsx, app/wifigate-setup.tsx |
| OTA | README_OTA_FIREBASE.md, fota/publicFota.ts |
| storage | storage.ts |

### BLE commands (SecureCommandName)

OPEN  TOGGLE_OPEN  TOGGLE_CLOSE  AUTO_OPEN
OPEN_GUEST  TOGGLE_OPEN_GUEST  TOGGLE_CLOSE_GUEST
PUSH_BUTTON_HOLD  PUSH_BUTTON_RELEASE
ADD_USER  ADD_ADMIN
WIFI_OPEN  OTA_OPEN  GET_FW_VERSION
GET_AUTO_OPEN_STATUS  CHECK_AUTO_OPEN_RSSI  GET_GATE_COMMAND

### gateCommand values in the response

1=PULSE  2=PULSE_AFTER_PULSE  3=PULSE_AFTER_PULSE_AFTER_PULSE  4=TOGGLE  5=PUSH_BUTTON

### Nordic UART UUIDs

Service    6e400001-b5a3-f393-e0a9-e50e24dcca9a
RX (write) 6e400002-b5a3-f393-e0a9-e50e24dcca9a
TX (notify)6e400003-b5a3-f393-e0a9-e50e24dcca9a

────────────────────────────────────────────────────────────

*This document was written by reading the source code. When something in the code changes (especially secureProtocol.ts, linkService.ts, gatesStore.ts) — update this too.*